#### Camp dresser

###### Equipment

- 6 x staves 1.8m long

- 2 x staves 0.5m long

- Sisal

- 2 x pegs

- Roll top table or more short staves

## Method

- Select the two straightest staves for the work top.  Using the remaining staves, make two identical A frames ensuring the horizontal bars are at waist height.  Use a sheer lashing at the top and square lashings on the horizontal staves. (a)

- Lash the other long staves on top of the short horizontal staves using square lashings.  If you are using a roll top table, make sure that it will sit on these staves without falling through. (b)

- If you have a square or rectangular washing up bowl it can be supported in the dresser by two additional short staves. (c)

- The rest of the dresser should be covered with either the table top or more short staves.

- To ensure that the dresser is quite stable, drive a peg into the ground at each end of the dresser and run a guy line from one peg, across the A frames to the other peg.  Secure the guy line to the pegs with a mooring hitch or round turn and two half hitches and to the A frames with clove hitches.

Camp Dresser