# Basic Camp Gateway

# Equipment

- 8 x light spars 2.5m (8ft) long

- 2 x light spars 2m (6ft) long

- 4 x light spars 1.5m (4ft) long

- Sisal

- Large pegs or pickets

- Troop name board

# Method

- The sides of the gateway are made from two narrow trestles. As they will not be carrying a great load they should only need a single diagonal brace, which saves time.

- The sides are held upright by an additional spar lashed to the middle of the top horizontal spar with a square lashing.  The position of this outer leg should be adjusted so that the sides lean outwards slightly and can stand on their own.

- Lash the two remaining spars across the top of the gateway and hang your name board from the front.

# Useful hint

Use a picket driven in at a 900 angle to the bottom of the outer leg and make fast with a square lashing.