# Simple Suspension Bridge

Equipment

- 4 x 5m (16ft) spars

- 2 x 3m (10ft) spars

- 4 x 2.5m (8ft) spars

- light spars for ladder rungs

- lashing lengths

- Long lengths of rope to span bridge and for rope ladder

- Large pegs or pickets

# Method

- Start by making a tripod on each side of the stream/gully using two 5m (16ft) spars and a 3m (10ft) spar that will be the back leg.  The actual lengths of the spars will depend on the depth of the stream etc.  Use either a figure of eight or a tripod lashing.

- Lash horizontal spars across the long legs to form and A frame.  They should be positioned so that they will be slightly higher than the bank when the legs are placed in their final positions.

- Place the longer legs in the stream so that they have firm footings and are on line.  Pull the shorter legs back until they are stable.

- Drive a picket into the ground alongside and at right angles to the back spar and lash the two together.

- The suspension rope can be tied off to the top of each tripod or a longer rope can be used and passed across the top of the tripod and tied off to the pickets.

- A rope ladder the width of the stream is constructed and tied between the A frames.  The centre span needs support from the top rope every metre or so.

- Two short ladders using the 2.5m (8ft) spars are constructed and placed either side of the bridge, leaning on the

cross spars of the A frame.

- Fit a hand rope if

found to be necessary.