# Camping Platform

In the 1970s there were a large number of Groups taking up a challenge by John Sweet to build multi-storey camping platforms.  This is a basic design which could be adapted and extended to suit the available materials.

Equipment:

17 x 6m (18ft) spars

9 x 4m (12ft) spars

2 x 3m (10ft) spar

10 x 2.5m (8ft) spars

2 x 2m (6ft) spars

10 x 1.2m (4ft) spars

2 x 1m stakes

4 long ropes for guys

51 lashing lengths

4 tent pegs

mallet

Method:

- Prepare 2 identical frames using 2 x 4m (12ft) spars joined by 3 2.5m (8ft) spars – one at the top, one about 1.2m (4ft) from the first and the third about 0.5m (1ft 6ins) from the butt.  Use a 3m (10ft) spar as a bracing between the lower two cross members. Tie guy ropes on either side of the frame, just above the middle cross member. (diagram A)

- Prepare a third frame similar to the first two but without the top cross member.

- Raise the first two frames approx 5m apart and put the guys in temporarily and tie ropes between the cross members front and back.

- Place the third frame between the first two, a little under 2m from one of the uprights.

- Lay 2 x 6m (18ft) spars on the bottom cross members of the frames.  Adjust the distance the frames are apart if necessary and re-guy.  Lash these two 6m (18ft) spars to the frames, one on either side.

- Then repeat using another 2 x 6m (18ft) spars and resting them on the centre cross members. Lash them in place.

- Cross brace the back of the platform using 1 x 2.5m spar and 1 x 3m (10ft) spar.

- Lash a 4m (12ft) spar vertically to the front of the frame approx 1.5m (5ft) from the centre support frame.

- Complete the platforms by lashing 6 or more 6m (18ft) spars between those already in place at each level.

- Prepare a ladder using   2 x 4.0m (12ft) and 10 x 1.2m (4ft) spars and lean it against the top platform between the two vertical spars.

- Lash the final 6m (18ft) spar to the top of the frames to create a handrail along the back of the frame.

- Lash two 2m (6ft) spars to the top of the front of the frame either side of the ladder to complete the handrail.

- Drive in 2 x 1m stakes either side of the foot of the ladder and lash 2 x  1.2m (4ft)  from the top of the stakes to the verticals either side of the top of the ladder to provide handrails.

- Cover the platform with any suitable material to make it more comfortable – planks of wood, bracken etc then pitch your tents.  You should be able to get 2 basic 2/3 person dome tents on each level.