A light pole bridge

This bridge is only suitable for shallow streams with very little current in them but can be extended to cross any distance by adding extra trestles to support more planks.

Equipment:

- 18 x staves or light poles  2m (6ft) long

- 3 x planks approx 3m (10ft)

- 34 lashing lengths

- 4 large pegs

Method:

- Two trestles should be prepared, one on either side of the stream if possible.  The upright poles must lean inwards for stability and care needs to be taken with the height of the top crossbar which, when the trestle is standing in the stream, should be level with the side of the bank on which the plank will rest.

- Cross bracing is lashed in place and where these two spars cross they are joined with a diagonal lashing.

- Two spars are lashed to the top crossbar so that, when the trestles are standing upright, these spars are horizontal and form the sides of the bridge.

- Lower the trestles carefully into the stream ensuring that the legs are sunk firmly into the mud.  The sides of the bridge should rest on the bank and pegs should be driven in alongside them.  The sides should be firmly lashed to the pegs.

- A plank should then be lowered onto the centre of the top crossbar of the trestle and tied in place.

- The centre of the bridge is then completed by lashing a further stave on each side of the bridge and then passing the third plank across between the trestles and tying it in place.

- For safety’s sake you may prefer to add a handrail - either a rope tied between the top of each trestle if they are high enough, or by lashing short uprights to the trestles to which additional  staves can be attached.