Swinging Derrick

# Equipment

- 3 x spars 3.6m long

- 5 x spars 2.5m long

- 2 x short spars 1m long

- lashing lengths

- additional ropes for guylines

- 1 x Handy Billy

Method

- The jib is made from 2 z 2.5m spars and 1 x1m spar.  Lay the long spars side by side and lash the thinner ends together with a sheer lashing.  Pull the butt ends about 15cm apart and lash the short spar across them using square lashings.

- Use the remaining 3 x 2.5m spars to make an A frame.

- Make a tripod by laying the 3 x 3.6m spars together and sliding the middle one along for about half its length.  Join the spars together with a figure of eight lashing.

- Attach the double block of the Handy Billy to the middle spar just above the lashing.

- Raise the tripod and adjust the legs so that the middle spar is vertical.

- Lash the prepared A frame to the legs of the tripod using square lashings.

- Attach the jib to the vertical spar with rolling hitches.

- Make a simple seat with the remaining short spar and fix it to the top of the jib.

- Attach the single  block to the jib and fit a pair of guy lines.

- Depending on what or who is being lifted, the derrick will require additional counter weights on the back of the A frame.  We tend to use three or four Scouts who are given instructions not to get up while it is in use.

Swinging Derrick