# The Long Range Bolt Shooter

Some years ago a Patrol of Scouts built a giant catapult and fired a stave an incredible 99 yards (90.5m).  Fortunately they had allowed plenty of room.

# Equipment

5 x 2.4m (8ft) spars

1 x 1.8m (6ft) spars

3 x 1m (3ft) spars

5 pickets

6 lashing lengths

4 x 5m (15ft) ropes for guys

1 single pulley

1 double pulley

1 x 15m (50ft) rope to suit pulleys

3 – 4 old car inner tubes  (depending on size)

Sisal

# Method

- Construct an A frame using 2 x 2.4m (8ft) and 1 x 1.8m (6ft) spars.

- Attach the guys (2 x 5m (15ft) ropes) to the top of each side of the A frame using round turns and two half hitches.

- Join the inner tubes together into a chain by binding them using plenty of sisal.  Hang the two ends of the tyre chain over the top of the A frame spars.

- Stand the A frame up, heel in the butts and then drive in the pickets.  The guys should be adjusted so that the A frame leans at approx 20o to the vertical.

- Set 2 x 1m (3ft) spars into the ground vertically approx 2.4m (8ft) from the A frame and parallel to it.

- Lash the third 1m (3ft) spar across the previous two about  0.3m (1ft) down from the top.

- Lay one of the 2.4m (8ft) spar in the crutch of the A frame and lash the other end (the butt) on top of the horizontal 1m (3ft) spar.

- Lay the other two 2.4m (8ft) spars alongside the first with one end in the crutch of the A frame and the other resting on the horizontal bar.  Join the three together at the lower end using a figure of eight lashing.

- Drive in the remaining picket in line with the sloping spars and about 1.8m (6ft) away from the support for the butts of the 2.4m (8ft) spars.

- Rig the pulleys and the 15m (50ft) rope into a handy billy.  Attach one end the handy billy to the centre of the tyre chain using sisal and the other end to the fifth picket.

# To fire the bolt shooter

- Prime the firing mechanism by tensioning up the handy billy and then make off to the rear picket.

- Load a light spar or stave.

- Check that the coast is clear and everyone is behind the line of fire.

- Cut the sisal.