Swing Boat

# Introduction

This project has the advantage that it is great fun to use when finished but it requires very good lashings and secure guying to remain stable during enthusiastic use.

# Equipment

- 6 x spars 3.6 metre (12 ft) long

- 6 x spars 3.0 metre (10 ft) long

- 1 x scaffold board 2.5 metre (8 ft) long

- 22 lashings

- 4 x ropes 8.0 metre long for guys

- large pegs or pickets

# Method

(a)

- 2 x A frames are constructed using 2 x 3.6 metre and 1 x 3.0 metre spar.

- The top of the sheer lashing needs to be approximately 1 metre below the top of the long spars.  The horizontal spar needs to be approximately 450 mm above the butts (a).

- Join the A frames together using the remaining 3.6 metre spars (b).

The lower spars need to be as low as possible, otherwise they may prove to be a hazard to the swing boat.

Lay the structure on its side. Join the tops of the A frames by lashing 3 metre spars to them and then, at right angles, lash the last 3 metre spar as a fulcrum across the centre  ( c).

(b)

Hint:   lean the A frames in slightly at the top to make the structure more stable.

- Tie two of the lashings to each end of the spar between the crutches of the A frames using a round turn and two half hitches.  These will support the swing boat.

- Tie a rope to each end of the fulcrum spar.

- Stand the structure on its feet.

- Suspend the plank from the four hanging ropes.  It may be necessary to notch the plank, or nail blocks onto it to prevent the ropes slipping.

- Peg out main guys.  Drive in pegs near the feet of the A frame and lash to the structure for stability.

USEFUL HINT

Whilst the structure is laid on its side it is a good idea to tie on four main guys.

Swing Boat