The Breckland Ditch Dodger

Equipment

approx 8 x 2.4m (8ft) spars

2 x 1.8m (6ft) spars

4 x 1.5m (5ft) stout pickets

2 x 1.2m (4ft) stout pickets

1 maul or sledge hammer

- Drive the pickets firmly into the ground in groups of 3 - 2 x 1.2m and 1 x 1.2m pickets.  There should be 2 groups on each side of the ditch, at right angles to it.  They should be approx, 1m (3ft) apart with the shortest stake in the centre.

- Place the 1.8m (6ft) spars between the groups of pickets, resting on the centre, shorter picket.

- Lay the 4m (12ft) spars across the ditch and on the 1.8m spars.

- If necessary, brailing pegs can be tapped between the 4m spars to prevent them moving.